Monitoring of LMOs released into the environment

INTRODUCTION

Monitoring, in the context of this guidance, refers to measures, undertaken after an LMO is released into the environment, that aim at detecting the occurrence of changes (e.g. in the receiving environment(s) or in the LMO) that could lead to adverse effects on the conservation and sustainable use of biodiversity, taking into account risks to human health. Adverse effects may be direct or indirect, short or long-term, immediate or delayed, and occur at various ecological levels and biological processes or at various stages in an organism’s life cycle or food chain.

Provisions in the Protocol that are relevant to monitoring are laid out in article 15 on “Risk Assessment”, paragraphs 8(e) and (f) of annex III, which states that the risk assessment methodology may entail “a recommendation as to whether or not the risks are acceptable or manageable, including, where necessary, identification of strategies to manage these risks” and “where there is uncertainty regarding the level of risk, it may be addressed by requesting further information on the specific issues of concern or by implementing appropriate risk management strategies and/or monitoring the living modified organism in the receiving environment”. Further, article 16 on “Risk Management” states “measures based on risk assessment shall be imposed to the extent necessary to prevent adverse effects”, and Parties shall “establish and maintain appropriate mechanisms, measures and strategies to regulate, manage and control risks identified in the risk assessment provisions”, and “endeavour to ensure that any living modified organism, whether imported or locally developed, has undergone an appropriate period of observation that is commensurate with its life-cycle or generation time before it is put to its intended use”.

Additional provisions that are relevant to the monitoring of LMOs are elaborated in the Protocol’s parent treaty, the Convention of Biological Diversity (CBD). According to article 7 of the CBD on “Identification and Monitoring”, Parties shall identify and monitor components of biological diversity important for its conservation and sustainable use.

According to these provisions, monitoring stands at the interface between risk assessment, risk management and decision-making, and may serve a range of purposes within these procedures.

In relation to the risk assessment, as seen in the Roadmap, uncertainties encountered during a risk assessment may affect the evaluation of overall risks and the conclusions of the risk assessment process. In cases where uncertainties identified in the risk assessment cannot be reduced by more information, monitoring strategies may be recommended to test risk hypotheses or scenarios in the event that a decision is taken to approve the LMO. As such, the results of monitoring activities may be used to evaluate the conclusions of the risk assessment process.

In relation to risk management, monitoring can identify the occurrence of events that could lead to adverse effects and in a timely manner for the implementation of appropriate response measures.

Monitoring may also be a tool to establish whether risk management strategies that have been enacted are being effectively implemented, or to identify adverse effects that were not dealt with in the risk assessment, because they could not be detected during the risk assessment process, e.g. long-term or cumulative effects.

In relation to the decision-making procedures, a country may require that monitoring be carried out after introduction into the environment if, for instance, there is uncertainty regarding the level of risk associated with an LMO.

Annex 1 provides a visual diagram to contextualize the use of monitoring within the entire risk assessment process.

OBJECTIVE AND SCOPE

The aim of this guidance is to provide a practical, comprehensive, science-based monitoring approach for LMOs intentionally released into the environment in line with the provisions and objective of the Protocol. It includes guidance on the design and evaluation of a monitoring strategy. While the focus of this guidance is on the scientific and technical aspects of designing and evaluating a monitoring strategy, considerations on the feasibility, availability of resources and technical challenges for successful implementation of a monitoring strategy are included.

Aspects related to monitoring in the context of decision-making, such as when and what types of monitoring should be enacted, and who bears the responsibility for its implementation and associated costs are policy issues that will not be addressed in this document.

This guidance focuses to the monitoring of environmental effects. The monitoring in the context of human and animal health, especially in the case of products used as food or feed would require different and/or additional approaches and are not covered in this guidance. Therefore the guidance here focuses considerations on human health in relation to incidental exposure to LMOs and their products directly by LMO handlers during their transport, use or processing.

It should be explicitly noted that this guidance relates to step 5 of the risk assessment process (see Roadmap) for intentional releases of an LMO into the environment. Unintentional introductions of LMOs into the environment and unintentional/illegal transboundary movements are outside of the scope of this guidance and dealt with under related articles in the Protocol.

The general provisions outlined here should be broadly relevant, yet not all specific provisions in this guidance will apply in each scenario, but should be determined on a case-by-case basis.

TYPES OF MONITORING

Monitoring can be grouped into two main types of activities: “case-specific monitoring” and “general surveillance”.

Case-Specific Monitoring (CSM) deals with potential adverse effects that were evaluated in the risk assessment of a specific LMO. CSM aims to provide further observational data, once the LMO is released into the environment, on whether any potential adverse effect identified in the risk assessment may occur, and it helps to verify if the conclusions of the risk assessment are accurate. Verifying the conclusions of the risk assessment through monitoring may be of particular value where predictive capacity of the information provided is limited (e.g. the scale or duration of release, degree of prior knowledge of the receiving environment, the exposure model considered, the experimental model chosen, etc.).

CSM tracks the occurrence of changes (e.g. in the LMO or the environment) that could lead to adverse effects that were identified in the risk assessment, particularly in step 1. These changes could, for instance, affect the likelihood or consequences (steps 2 & 3) of one or more potential adverse effects identified during the risk assessment and, therefore, impact the estimation of overall risk of the LMO (step 4). As a result, a CSM strategy has close links with the preceding steps of the risk assessment.

CSM may also be a useful tool when the level of uncertainty regarding the risks of the LMO is such that it could impact the conclusions and recommendations of the risk assessment process. In such cases, monitoring strategies can be recommended to compensate or reduce uncertainties identified throughout the risk assessment. Therefore, the identification and description of uncertainties arising in the risk assessment (see “Identification and consideration of uncertainty” in the Roadmap) provides important elements to determine whether a monitoring strategy is needed and how it could be designed.

General Surveillance (GS) monitors for adverse effects that were not identified or anticipated in the risk assessment. Such instances may be due, for example, to the complexity of the receiving environment, or to unknown modes of exposure to the LMO including strategies to monitor the occurrence of cumulative or combinatorial adverse effects.

GS can also be viewed as an important tool in risk management with respect to the safeguarding a country’s protection goals, such as in cases of threats that are considered serious or irreversible, but lack full scientific certainty of their occurrence.

In conclusion, a risk assessor may, on a case-by-case basis, and depending on whether the national biosafety legislation or policies have any requirements to this effect, make a recommendation for the implementation of one and/or the other type of monitoring, i.e. CSM and GS, and include a comprehensive plan for a monitoring strategy.

DEVELOPMENT OF A MONITORING STRATEGY

If a recommendation is made at the end of the risk assessment for the implementation of monitoring activities in the event that the LMO is introduced into the environment, this recommendation should be substantiated with a description of a scientifically rigorous and effective monitoring strategy. This monitoring strategy can utilize, as appropriate, either one or the two types of monitoring identified above (i.e. CSM and GS), and may include provisions to ensure the scientific quality and efficacy of the monitoring activities, and for reporting of monitoring data.

Regardless of the type of monitoring, i.e. CSM and GS, the design of a monitoring strategy should address a number of technical issues that supports the objectives of the Protocol, national protection goals, and contributes to informational needs under risk assessment, risk management, and/or decision-making. Further, the description of a monitoring strategy should be transparent and presented in sufficient detail ensure scientific quality.

When developing a strategy for CSM and/or GS, or evaluating a proposed monitoring strategy, the following should be considered:

- The identification and prioritization of protection goals, potential adverse effects and the choice of indicators and parameters for monitoring (“what to monitor?”);

- Identification and description of appropriate monitoring methods and establishment of baselines (“how to monitor?”);

- Duration and scale of the monitoring activities (“how long to monitor?”);

- Monitoring sites and regions (“where to monitor?”);

- The use of existing monitoring networks;

- Reporting of results from monitoring;

- Analysis of the feasibility and potential challenges associated with the implementation of the recommended monitoring strategy.

These points are further elaborated below.

1. The identification and prioritization of protection goals, potential adverse effects and the choice of indicators and paramaters for monitoring (“what to monitor?”)

Rationale for the identification and prioritization of protection goals and potential adverse effects:

The identification of potential adverse effects, indicators and parameters to be monitored will vary from case to case and will depend on risk hypothesis and scenarios that were established during the risk assessment and on the protection goals and biosafety legislation or policies of each country.

Under a case-specific monitoring or CSM, potential adverse effects should be principally derived from specific hypotheses or scenarios identified in the risk assessment (see Step 1-5 of the Roadmap) in a “bottom-up approach” [to be added to the “Use of terms” section].

Under a general surveillance or GS, the design of a monitoring strategy focused on potential effects that support identified protection goals, where no particular hypothesis for an adverse effect has been established in a “top-down approach” [to be added to the “Use of terms” section]. Further, a general surveillance or GS strategy may also include an analysis to identify adverse effects not covered in the CSM plan where hypotheses for testing under GS may be derived from further modelling, geo-spatial analysis, cause-effect scenario analysis, or general ecological knowledge.

There may be additional relevant adverse effects that are identified on the basis of hazard identification research, but not evaluated in the risk assessment (e.g. long-term impacts, food-web interactions, effects on human health from LMO handling). These types of effects may be addressed in by either CSM or GS, depending on the indicators or parameters selected.

Thus, it may not always be a straightforward choice of which types of potential adverse effects and what protection goals fall under CSM or GS. Depending on the circumstances, CSM and GS strategies may share similar parameters or indicators (e.g. when monitoring for possible cumulative effects of LMOs growing in the same area, measuring indirect or direct uptake of transgenic products in non-target organisms), Further, GS may begin as a hypothesis- or scenario-free observation where detecting causation between the observed effects and the LMO may lead to testable hypotheses through further investigation. In these cases, GS may help identify adverse effects that may be further approached with case-specific monitoring.

Points to consider regarding the identification of potential adverse effects or protection goals:

- Likelihood and consequences of a potential adverse effect identified in step 1 of the risk assessment (see Roadmap) to occur, in particular for CSM;

- Protection goals (e.g. protection of biodiversity, ecological function and ecosystem services) within the appropriate ecosystem spheres (land/soil, water) in the relevant environment, in particular for GS;

- Uncertainties that arose during the risk assessment process, in particular those related to specific risk hypotheses or scenarios (for CSM) as well as those that may affect the protection goals (for GS).

Rationale for the identification and selection of indicators and parameters:

Under a CSM strategy, the indicators (e.g. species, populations, groups of species, environmental processes, etc.) and parameters (i.e. a component to be measured in the observation of an indicator) chosen are those that can best detect changes that could lead to the potential adverse effects identified during the preceding steps of the risk assessment. On the other hand, since a GS strategy cannot be expected to evaluate effects arising in each sphere of the ecosystem (i.e. water, soil and air) and at every scale of interactions within them (i.e. species/populations, communities, habitats, ecosystems, etc.), the selection and prioritization of indicators and parameters to be used in a GS should be based on their potential to signal changes in the LMOs or reveal particular protection concerns (e.g. decline of a protected species). The protection goals and assessment endpoints within each National Party’s domestic legislation can inform which indicators may be relevant.

Points to consider regarding the identification and selection of relevant indicators and parameters:

- The potential of the indicator or parameter to signal possible LMO-induced changes;

- The breadth of distribution and abundance of an indicator and its level of exposure to the LMO;

- The importance of the indicator or parameter to key ecological processes and functions or to the identified protection goals;

- The potential of the indicator or parameter to reveal changes that could be an indicative of adverse effects;

- The level of difficulty involved in the sampling or identification of the indicator;

- The ability to establish relevant baselines with the indicator.

- The relation of the indicator or parameter to identified protection goals.

Annex 2 provides examples of indicators and protection goals that may be part of a monitoring strategy.

2. Identification and description of appropriate monitoring methods and establishment of baselines (“how to monitor?”);

Rationale for the identification and description of appropriate monitoring methods:

The choice of monitoring methods is largely dependent on the identification of potential adverse effects or protection goals, as well as indicators and parameters decided upon in the preceding step.

The description of the monitoring methodology includes the steps of collecting and analysing data. These may include, for example, methods for (i) sampling of biotic (e.g. of LMOs and/or indicator species) and abiotic (e.g. water, soil) components of the receiving environment, (ii) gathering information (e.g. questionnaires), (iii) generating data (e.g. analytical methods), and (iv) data analysis (e.g. statistical methods, procedures, and statistical significance requirements). The monitoring methodology should be described in sufficient detail to allow reproducibility, and be based as much as possible on the use of standardized methods (see also considerations on “Quality and relevance of information” in the Roadmap).

An important feature of the proposed monitoring strategy and its methodology is the degree to which results from various sites and regions can be compared (see “Monitoring sites and regions” below). Also, a monitoring strategy may be designed in such way that it is in line with existing monitoring programs, e.g. conservation, agricultural and environmental monitoring schemes. This is particularly true in the case of a GS strategy that utilizes already established programmes for the surveillance of broader protection goals. Harmonization of methods, data formats, and analytical approaches may facilitate the adaptability of monitoring methods performed by these programmes.

In describing appropriate methods, it should be considered that those most closely associated with the actual use of the LMO (e.g. farmer, land manager) may be the first to observe relevant changes. Therefore, the use of observations, descriptive studies, or questionnaires from those in the user-chain should be included where possible in the collection data for unanticipated effects as supplementary information, if appropriate.

Points to consider regarding the monitoring methods:

- The nature of the adverse effect to be monitored (e.g. whether short or long term, delayed or indirect);

- Methods for establishing relevant baselines;

- The scientific rigor of the sampling, analytical and statistical methods;

- The availability of standardized methods;

- The degree to which the methods will meet the objectives of the proposed strategy;

- Descriptive studies or questionnaires as supplementary information to the proposed scientific monitoring strategy;

- The adaptability of any existing already established programmes for the surveillance of broader protection goals (in the case of a GS strategy).

Rationale for the establishment of baselines:

The establishment of relevant baselines is a key element for detecting changes and inferring whether there is a causal link to the presence of one or more LMOs. The baseline should be described in the monitoring methodology in order to provide an accurate representation of the environment prior to its exposure to the LMO(s). In practice, the baseline is a measurement of the relevant indicators prior to the introduction of the LMO(s) in the likely potential receiving environment. While the data needed to establish a baseline may be readily available from previous studies, it may also need to be generated before the introduction of the LMO or in parallel, based on similar receiving environments that have not been exposed to the LMO(s).

Points of consideration for the establishment of baselines:

- The use of scientifically rigorous methods in constructing the baseline;

- The spatial scale over which to establish the baseline;

- Effects of spatial heterogeneity on the representativeness of the baseline in each of the compared scenarios (LMO vs. non-LMO);

- The breadth of potential spread related to the type of LMO.

3. Duration and scale of the monitoring activities (“how long to monitor?”)

Rationale:

The duration and scale of the monitoring will depend on the type of strategy (i.e. GS or CSM), adverse effects that are to be monitored (e.g. direct or indirect, immediate of delayed, short- or long-term), and type of LMO (e.g. short or long life cycles). The duration and scale of the monitoring strategy may further vary for each proposed parameter and/or methodology required to achieve relevant scientific information.

According to the Protocol, Parties “shall endeavour to ensure that any living modified organism, whether imported or locally developed, has undergone an appropriate period of observation that is commensurate with its life-cycle or generation time before it is put to its intended use”. In this context, monitoring can be used to generate data for risk assessments, for example, during field trials and small-scale releases. The results from a monitoring carried out during a small-scale release may, for instance, increase the scientific rigor of the conclusions of a risk assessment and help avoid the need for later risk management measures at a large-scale release or contribute to much more targeted, cost-effective monitoring strategies. When the risk assessment and monitoring is conducted in such a step-wise manner, monitoring of small-scale releases can also limit the consequences of an adverse effect as compared to the consequences should an adverse effect occur after a large-scale introduction into the environment.

Monitoring activities that require long periods of observation in order for changes to become apparent may pose a number of practical challenges, e.g. in consistency or availability of test sites or subjects over long periods, and empirical limitations (e.g. establishing a causal relation with any change identified in the monitoring program) that should considered in the monitoring strategy (see “Analysis of the feasibility and potential challenges associated with the implementation of the recommended monitoring strategy” below).

Points to consider:

- Different types of adverse effects (i.e. direct or indirect, immediate or delayed);

- Life-cycle and generation time of the LMO as well as its intended use;

- The variability of the monitored parameters through time;

- Unanticipated changes that may be difficult to predict or detect;

- Effects may become detectible only after a longer period of observation.

4. Monitoring sites and regions (“where to monitor?”)

Rationale:

Monitoring sites and regions should be selected on a case-by-case basis depending on the type of LMO or indicator (e.g. microorganism, plant or animal, its level of domestication, its life-cycle, its novel characteristics, etc.), the intended use of the LMO taking into account the associated management practices, and the likely potential receiving environment. The sites and regions to be monitored should be representative of the various biotic and abiotic characteristics of the likely potential receiving environment(s) where the LMO is to be released. The pathways of exposure of the environment to the LMO(s) (e.g. seed dispersal or movements of LM animals), as well as areas linked to protection goals (e.g. protected areas and centres of crop origin and genetic diversity or ecologically sensitive regions, may serve as basis for selecting sites to be monitored under both CSM and GS strategies that extend beyond the intended receiving environment where the LMO(s) may be introduced.

Relevant information regarding the sites and regions to be monitored include, for example, specific locations, their size and relevant characteristics of the sites may be included in the monitoring strategy.

Points to consider:

- Availability of reference sites and regions without the LMOs for a comparisons over the monitoring period;

- Dissemination and establishment of the LMO(s) in the likely potential receiving environment;

- Pathways through which the environment is likely to be exposed to the LMO(s);

- The biological and ecological behaviour of the indicators in the receiving environment for consistent detection and observation;

- Protected areas and centres of origin and genetic diversity or ecologically sensitive regions with specific protection goals, including the use of buffer areas in order to detect unintended presence or unexpected effects;

- The availability of existing monitoring networks operating within representative regions;

- Number of monitoring sites and regions sufficient to support statistical analysis of results;

5. The use of existing monitoring networks

Rationale:

The monitoring plan should specify the criteria for the selection of any existing monitoring systems and programs to be used supplying monitoring data. The suitability of such networks should be evaluated beforehand with respect to their potential to achieve the goals of the monitoring plans, taking into account, inter alia:

- Adaptability of existing monitoring to LMO monitoring indicators or parameters;

- The robustness of data generated possible to meet the monitoring objectives;

- The number and relevance of existing indicators for LMO monitoring;

- Representativeness of sites in number or distribution in relation to the intended receiving environment of the LMO release;

- The frequency of observation and methods employed;

- The long-term continuity of the monitoring sites;

- The capacity of the managing institution to collect, report and disseminate data derived from monitoring activities;

- Access to data before or beyond the timeframe of observation;

- Expertise and resources available to carry out the relevant monitoring activities.

6. Reporting of results from monitoring

Rationale:

The monitoring strategy may include a plan of how the results of the monitoring activities are to be communicated. Such a reporting plan can include, for instance, (i) the periodicity of the reports, (ii) a description of the activities undertaken, (iii) a scientifically rigorous analysis of the results including whether and what changes were detected in the indicators in comparison to the baseline data and reference sites and regions without the LMO(s), and (iv) conclusions and recommendations.

Specific considerations may also be included on what and how to report in the event that changes that could lead to adverse affects are observed during the monitoring activities. Such considerations may include, for example, how to establish a causal link between the LMO(s) and adverse effect(s) and an analysis of the consequences of the adverse effect(s).

A clear description of how the results may be reported can serve various purposes, such as to (i) provide feedback of the efficiency and efficacy of the monitoring activities in relation to the objectives set out in the monitoring strategy; (ii) to inform the need for changes to the monitoring plan and/or other risk management strategies, or for follow-up studies or risk assessments; and (iii) inform authorities of adverse effects or unintentional/illegal transboundary movements of LMOs.

Points to consider:

- Requirements regarding reporting of results from monitoring activities that are set out by the competent authority(ies) or in national biosafety regulations, if available;

- The LMO, including its potential adverse effects and overall risk, the intended use and the likely potential receiving environment as well as any other element that could affect the periodicity of reporting;

- The choice of methods, duration and scale, as well as sites and regions of the proposed monitoring activities;

- How to report changes (e.g. to indicators) observed during the monitoring that could lead to an adverse effect and any possible mitigation measure;

- Any potential challenge associated with the monitoring which could affect its implementation (see below);

7. Analysis of the feasibility and potential challenges associated with the implementation of the recommended monitoring strategy

Rationale:

In the development of a monitoring strategy, it may become apparent that the resources necessary for the implementation of the monitoring strategy may be unavailable or entirely lacking. Further, technical and analytical challenges may be highlighted. In such cases, an analysis of the capacities that may be required for the successful initiation, maintenance and completion of any recommended monitoring strategy should be considered.

Points to consider:

- Possible methodological weaknesses in the monitoring plan to provide statistically meaningful data;

- Whether monitoring sites are representative of all likely potential receiving environments;

- Challenges in observing adverse effects;

- Challenges for establishing cause-effect relationships (causalities) between the LMO(s) and adverse affects;

- Possible difficulties in the interpretation of monitoring results and scientific considerations on what environmental changes should be further investigated;

- Costs and capacities to conduct an effective monitoring;

- Challenges in the utilization of existing monitoring programs or methodologies;

- Unanticipated results that may require adaptation of the monitoring activities (e.g. changes in or inclusion of new parameters or indicators).

Annex 1

Recommendation of monitoring strategies in relation to risk assessment, decision-making and implementation of risk management under the Protocol

Annex 2

Examples of monitoring subjects indicators and monitoring methods in relation to protection goals

| Type of monitoring (CSM or GS) | Protection goal(s) / Objective | Subjects/Indicator(s) | Example(s) of monitoring methods |
| --- | --- | --- | --- |
| CSM | Reduction of level on uncertainty of potential effects identified in the RA | Target organisms, Non-target organisms, environmental parameters, etc. | • Confirming host-range effects of target transgenic proteins, resistance development, • Confirming exposure routes or levels |
| CSM | Impact on assessment endpoints or related indicators identified and evaluated in the RA | Target organisms, non-target organisms, environmental parameters, etc. | • Presence and population levels of key selected NTOs • Food web and predator/prey interactions of key selected NTOs at different trophic levels |
| CSM | Confirmation of in vivo exposure levels | Non-target organisms, etc. | • Direct or indirect uptake/exposure of NTOs to transgenic pesticidal proteins • Existence of weed species in herbicide tolerant (HT) fields • Accumulation of transgenic products in the soil |
| CSM | Impact on production systems in relation to sustainability | Functional organisms, key environmental services, etc. | • Pollination impacts • Pest control efficacy |
| CSM | Monitoring for scale-dependent effects | Wild and weedy relatives, HGT candidates | • Persistence of DNA or transgenic products in the soil • Frequency of gene transfer potential |
| CSM | Efficacy of risk management strategies | Case-specific | • Efficacy of refugia strategies to delay resistance development of pesticide-producing crops by testing susceptibility of target pests • Recording weed populations in HT crop fields or adjacent areas |
| GS | Conservation of terrestrial faunal biodiversity | Vertebrates (mammals, birds, etc.), invertebrates (arthropods, fungi) with a focus on beneficial/functional organisms or protected species | • Abundance and population changes • Resistance development • Effects of agrochemical usage associated with the LMO in indicator species • Developmental and fitness changes (direct and indirect) in indicator species • Host range or key behavioral changes in indicator species • Dissemination changes for the LMO • Changes in pest prevalence or pathology • Landscape alterations |
| GS | Conservation of terrestrial floral biodiversity (including genetic diversity) and ecosystems | Primary producers (e.g. plants) with a focus on beneficial/functional organisms and important sources of genetic diversity, and protected species | • Outcrossing/hybridization with wild or weedy relatives • Plant population dynamics and changes • Effects of agrochemical usage associated with the LMO • Fecundity and fitness effects • Dispersal, establishment and persistence • Landscape alterations |
| GS | Soil quality and functional processes | Soil microbes and invertebrates (e.g. bacteria, fungi, and arthropods) particularly those providing key soil ecological services (nutrient cycling and decomposition) | • Population changes • Gene transfer frequencies • Organic compound changes • Effects of agrochemical usage associated with the LMO • Soil fertility changes • Changes to degradation processes • Soil erosion and compaction changes |
| GS | Conservation of aquatic biodiversity (including genetic diversity) and ecosystems | Aquatic species (e.g. fish, arthropods, algae, plants, mammals) with a focus on beneficial/functional organisms and important sources of genetic diversity, and protected species | • Abundance and population changes • Effects of agrochemical usage associated with the LMO in indicator species • Developmental and fitness changes (direct and indirect) in indicator species • Host range or key behavioral changes in indicator species • Dissemination changes for the LMO • Changes in pest prevalence or pathology • Habitat alterations • Outcrossing/hybridization with wild or weedy relatives • Fecundity and fitness effects • Dispersal, establishment and persistence |
| GS | Air quality and air pollution prevention | Organic/inorganic pollutants, volatiles, greenhouse gas/C02 concentrations, pollen loads, etc. | • Particulates analysis • Ozone and SO4 concentrations • Pollen counts |
| GS | Water quality and water pollution prevention | Physical and chemical pollutants in water, etc. | • Nutrient levels • Pollutants: pesticides, herbicides, etc. • Emission of transgenic product to water • Anoxia |
| GS | Plant health | Plant diseases, pests and weeds, etc. | • Incidence of disease, pests and weeds • Pesticide usage |
| GS | Human health (e.g. LMO handlers) | Handlers of LMOs or their products (e.g. farmers, research technicians, mill workers, etc.) | • Exposure analysis • Screens for toxic or immunogenic effects • Epidemiological surveys |
| GS | Agroecological sustainability | Floral and faunal indicators of functionality (pollinator populations, beneficial plant communities), non-renewable input levels, etc. | •Abundance • Foraging behaviors and pollination levels • Soil indicators |
| GS | Socioeconomic aspects | Agricultural methods or production systems, etc. | • Changes in the spectrum/abundance of diseases, pests, or beneficial organisms • Reduction in effectiveness of target trait or management practices • Changes in cultivation practices |

Sources:

Food and Agriculture Organization of the United Nations. (2011). Biosafety resource book. Rome: FAO, Module B: Ecological Aspects and Module D: Test and Post-Release Monitoring of GMOs.

VDI-Guideline 4330 Part 1: Monitoring the ecological effects of genetically modified organisms, Genetically modified plants, Basic principles and strategies, 2006.

EFSA Panel on GMO; Scientific Opinion on guidance on the Post-Market Environmental Monitoring

(PMEM) of genetically modified plants. EFSA Journal 2011;9(8):2316. [40 pp.]

See CBD, Article 7

See Roadmap for Risk Assessment, ”Overarching issues” and Step 5 of ”Conducting the Risk Assessment”.

This may be particularly useful for releases of approved for experimental/field testing purposes where the LMO is evaluated incrementally over successive scales of release, but also applicable for large scale environmental releases.

See Cartagena Protocol, Article 10 p6, Article11 p8, Article 12 p1

See Roadmap and Annex III p8 (x)

See Roadmap, ”Overarching issues”

Article 16, paragraph 4.

Roadmap for Risk assessment, Step 1 Rationale

See articles 17 and 25.