Monitoring of LMOs released into the environment

Version of 22 February 2012

INTRODUCTION

This document builds on and complements the Roadmap for Risk Assessment of Living Modified Organisms.

In the context of this guidance, monitoring of LMOs refers to the systematic observation, collection, and analysis of data undertaken based on the risk assessment and following the release of an LMO into the environment, and in accordance with the objective of the Protocol.

Paragraph 8(f) of Annex III, states that “where there is uncertainty regarding the level of risk, it may be addressed by requesting further information on the specific issues of concern or by implementing appropriate risk management strategies and/or monitoring the living modified organism in the receiving environment”. As such, monitoring is one of the possibilities to reduce uncertainty related to the level of an identified risk of an LMO. In accordance with the terms of reference for the AHTEG, this document provides guidance on “monitoring of the long-term effects of living modified organisms released in the environment”..

Monitoring may help detect changes related to adverse effects, in a timely manner, before the consequences are realized, and may inform the need for appropriate response measures (e.g. changes to risk management strategies, emergency response measures, a new risk assessment, or re-evaluation of prior decisions).

OBJECTIVE AND SCOPE

The present document aims at providing conceptual, science-based and practical guidance for monitoring of changes that could be related to adverse effects of LMOs released into the environment and that could affect the conservation and sustainable use of biological diversity, taking into account risks to human health. This guidance may be applicable to all classes of LMOs, and scales of release into the environment (e.g. small- and large-scale releases).

Monitoring of potential adverse effects to human health in the context of environmental risk assessment is included in this guidance.

Issues related to the decision as to whether or not monitoring should be implemented, or who bears the responsibility for its implementation and associated costs, are not addressed in this document.

MONITORING AND ITS PURPOSES

Monitoring is done in a case-specific manner to address uncertainties related to level of risk identified in a risk assessment. When recommended in step 5 of Annex III of the CPB, the case-specific monitoring reflects the considerations in the earlier steps of the risk assessment and the considerations on uncertainty with regard to the level risk of the LMO.

The implementation of case-specific monitoring in conjunction with an approved release of an LMO may provide observational data about specific effects of the LMO on relevant components of the ecosystem.

Case-specific monitoring of the environmental release may be done for different purposes, depending on the type (e.g. experimental or commercial), duration (e.g. short- or long-term) and scale (e.g. small- and large-scale) of release, as well as on the identified uncertainties regarding the level of risk or its management:

• Monitoring during experimental, short-term and/or small-scale environmental releases

Monitoring can generate data during experimental, short-term and small-scale releases in order to provide data for future risks assessments that may involve a larger scale of release of the same LMO. When environmental releases of an LMO are conducted in a step-wise manner, monitoring at smaller scales may increase the scientific strength or certainty of risk assessments for subsequent larger scale releases.

• Monitoring during long-term and/or large-scale environmental releases

During long-term and large-scale releases of an LMO (e.g. for commercial purposes), monitoring may be conducted in order to address remaining uncertainties regarding the level of a risk identified in the risk assessment, i.e. to confirm that assumptions of the risk assessment are accurate once the environmental release has taken place.

• Monitoring to evaluate the efficacy of specific risk management strategies

In cases where risk management strategies are implemented along with an environmental release, monitoring may be used to evaluate the effectiveness of these risk management strategies.

Case-specific monitoring may also be undertaken to detect whether there are changes related to potential adverse effects that were identified but not addressed in the risk assessment (e.g. effects such as long-term, tri-trophic, cumulative, as well as changes to management practices and effects to human health).

.

In addition to case specific monitoring for LMOs, where the trigger for monitoring lies in the result of the  risk assessment, some countries may also have existing general monitoring practices that are conducted in the context of article 7 of the CBD.  This kind of general monitoring is conducted unrelated to the release of an LMO, but addresses more general questions related to the conservation and sustainable use of biological diversity.

Data collected through these monitoring activities may be taken into account.In this approach, the trigger for further action is an observed adverse phenomenon in the environment, for example a significant reduction of a protected species. When such a phenomenon has been identified, authorities typically explore the possible causes of the observed phenomenon.

The next step is to identify changes in the environment that are correlated in time and place with the phenomenon and that could plausibly be its cause. These changes could for example be in agricultural management, such as change in pesticide use.

If one of the identified correlated changes in the environment is the introduction of an LMO that could plausibly lead to the observed phenomenon, a specific hypothesis will need to be developed and tested in order to establish a scientifically valid causal relationship.  Data for establishing such a relationship may be obtained from a scientifically valid study designed to test the hypothesis, or through case specific monitoring.

DEVELOPMENT OF A MONITORING PLAN

A monitoring plan is developed when the recommendation of a risk assessment calls for monitoring activities to be carried out in conjunction with the environmental release of the LMO. In such cases, the competent authority(ies) or the entity responsible for the risk assessment may outline the requirements of a monitoring plan (including the reporting of monitoring data). The monitoring plan should be transparent, of scientific quality and presented in sufficient detail so that the relevance of the data can be appraised.

If the monitoring plan is to be developed by the notifier, it may be evaluated by the competent national authority and may be subject to modification before a decision for release is granted. It is important to consider that the proposed monitoring activities should be commensurate with the uncertainty regarding the level of risk identified in the risk assessment.

Information relevant for developing the monitoring plan may be available from the risk assessment and, if applicable, from previous monitoring activities, including those from other countries. For example, the choice of indicators and parameters may often be derived from the context and scoping phase of the risk assessment (See Roadmap, “Setting the context and scope”). The scientific and technical details of the specific LMO, including detection methods, would be available from the information required for conducting the risk assessment as outlined in Annex III.

This guidance focuses on the development of a monitoring plan to address uncertainty regarding the level of risk of an LMO identified in the risk assessment, When developing (or evaluating) a monitoring plan, the following may be considered:

- Description of how monitoring data would address the uncertainty regarding the level of risk of an LMO (“why to monitor?”);

- Choice of indicators and parameters for monitoring (“what to monitor?”);

- Monitoring methods, including the establishment of baselines and the duration of monitoring (“how to monitor?”);

- Monitoring sites and regions (“where to monitor?”);

- Reporting of monitoring results (“how to communicate?”).

The sections below address these issues in terms of rationales and points to consider.

1. Description of how monitoring data would address the uncertainty regarding the level of risk of an LMO (“why to monitor?”)

Rationale:

The monitoring plan may differ according to the uncertainties regarding the level of risk identified in the risk assessment, as well as monitoring of the efficacy of risk management measures. The monitoring plan should be described in such a way that it will contribute to achieving its expected outcomes.

Points to consider:

- Uncertainties regarding the level of risk of the LMO;

- Identified causal pathways from the LMO to potential adverse effects, if applicable, in relation to the risk hypothesis;

- Uncertainties related to the duration and scale of the release;

- Uncertainties related to the effectiveness of the implementation of risk management measures.

2. Indicators and parameters for monitoring (“what to monitor?”)

Rationale:

The selection of indicators and parameters to be monitored will vary from case to case, depending on the LMO, the characteristics of the receiving environment, and specific risk scenarios established during the risk assessment (see the Roadmap).

The indicators (e.g. species, populations, groups of species, environmental processes, etc.) and parameters (i.e. a component to be measured in the observation of an indicator) chosen are ideally those that can reliably signal changes relevant to potential adverse effects and address uncertainties in the level of risks.

[Annex 2 provides examples of indicators and parameters that may be part of a monitoring plan.]

Points to consider:

- The potential of the indicators and parameters to signal changes relevant to potential adverse effects, in particular, before the consequences are realized;

- Characteristics of the indicators, as well as the distribution and abundance of those indicators that are species and, if applicable, their level of exposure to the LMO;

- Variability of the parameters to be measured;

- The usefulness of the chosen indicators and parameters to establish relevant baselines, including reference points;

- The importance of the indicators and parameters to relevant key ecological processes and functions or to the identified protection goals;

- Whether sampling and analysis would be easy or difficult and how these would affect the choice of indicators and parameter.

3.  Monitoring methods, reference parameters and duration of monitoring (“how to monitor?”)

a) Selecting monitoring methods

Rationale:

Monitoring methods are largely dependent on the indicators and parameters chosen in the preceding step and their ability to address uncertainty regarding the level of risk and to signal adverse effects. The selection of monitoring methods should also take into account their level of sensitivity and specificity needed to detect changes in the indicators and parameters.

The description of the monitoring methodology includes the means for sampling and observing indicators and parameters, and analyzing the resulting data. Appropriate methods, observations, descriptive studies, or questionnaires may be useful in the collection of data for monitoring, including questionnaires addressed to those who are exposed to the LMO. For ecological issues, or effects occurring outside of the receiving environment, additional knowledge and tools may be required to gather relevant data.

Harmonization of methods, data formats, and analytical approaches facilitates the comparison of results from monitoring. When the use of existing monitoring networks is to be considered, the monitoring plan should specify the criteria for their selection and utilisation.

Points to consider:

- Relevance of the monitoring methodology to generate information to address uncertainty related to the level of risk;

- The nature of the change to be monitored (e.g. whether short- or long-term, delayed or indirect, cumulative, etc.);

- Relevance, suitability and adaptability of existing broader monitoring schemes, as well as the accessibility to those data, in the context of broader environmental monitoring for unanticipated adverse effects that were not identified in the risk assessment;

- The specification of the ranges or degrees of changes in a parameter or indicator to signal an adverse effect;

- The scientific quality of the sampling, analytical and statistical methods to be employed;

- The availability of relevant standardized methods, and whether and how these could be taken into account;

- Whether methods are adequate to meet the objectives of the proposed monitoring plan;

- The use of descriptive studies or questionnaires, taking into account their replicability and verifiability;

- Findings of the ongoing and/or other monitoring activities, if relevant;

- Relevant local, regional and international monitoring practices.

b) reference data

Reference points, which may be baselines, are necessary for observing and analysing changes during monitoring. In practice, the reference point is a measurement of the relevant indicators and parameters in the likely potential receiving environment, or in a comparable environment. Therefore, the reference point should be described in the monitoring methodology in order to verify that it accurately represents the environment where the LMO will be released. Natural and human induced variation that may occur in reference data should be taken into account when analysing monitoring data.

Points of consider:

- The scientific quality of methods used for generating reference data;

- The appropriate spatial scale over which to establish thereference data;

- Effects of temporal and spatial variation (i.e. human induced or natural variation);

- The scale of potential spread of the LMO.

c) Duration of monitoring

Rationale:

The duration of the monitoring, including the frequency of observations necessary, is chosen on a case-by-case basis and will depend on the type of changes that are to be monitored (e.g. immediate or delayed, short- or long-term), type of LMO (e.g. short or long life cycles, transgenic traits introduced), or duration of proposed environmental release. The duration of monitoring may be changed, if appropriate, on the basis of the results of on-going monitoring activities.

Points to consider:

- The duration necessary for changes in a parameter related to the adverse effects to likely become apparent;

- Life-cycle and generation time of species to be used as indicators;

- Life-cycle and generation time of the LMO as being used in the environment;

- Whether variability in the monitored parameters over time could affect the results of the monitoring;

- Potential for environmental changes.

4. Monitoring sites (“where to monitor?”)

Rationale:

Monitoring sites are selected on a case-by-case basis depending on the parameters and indicators that will be used in the monitoring and the likely potential receiving environment, as well as the intended use of the LMO, and taking into account the associated management practices. The likely potential receiving environment may include areas that extend beyond the intended receiving environment where the LMO may be introduced.

Relevant information regarding the sites to be monitored include, for example, specific locations, their size and relevant environmental characteristics.

Points to consider:

- Dissemination and establishment of the LMO in the likely potential receiving environment;

- The type of LMO as well as indicators and parameters to be monitored and, in case of indicators species, their biological or ecological characteristics and life cycles;

- Appraisal of suitable, relevant reference sites where the LMO is not present for a comparison over the duration of the monitoring, if applicable;

- Pathways through which the environment is likely to be exposed to the LMO(s);

- The distribution patterns, including seasonal distribution (e.g. migration), of the selected indicator species in the receiving environment for consistent detection and observation;

- Appraisal of protected areas and centres of origin and genetic diversity or ecologically sensitive regions, particularly in the context of monitoring the presence of LMOs;

- The appropriate number of monitoring sites sufficient to support meaningful statistical analysis;

- The continued availability of the monitoring sites throughout the duration of monitoring;

- Current management practices and possible changes to those practices over the duration of monitoring.

5. Reporting of monitoring results

Rationale:

Reporting of monitoring results serves four main objectives: i) to inform competent authorities of any changes that could be related to adverse effects, ii) to provide feedback as to whether the monitoring activities have been carried out in a manner that meets the intended objectives set out in the monitoring plan, iii) to indicate, if appropriate, the need for changes to the monitoring strategy and/or other risk management strategies (or for follow-up studies or risk assessments), and iv) to recommend, if appropriate, the re-evaluation of a decision and the necessity of any emergency measures.

The reporting of monitoring activities may be communicated in different forms depending on the target audience. Since monitoring is both a scientific and regulatory activity, the report should clearly describe how the scientific results relate to the original regulatory need for monitoring. From the report, the regulatory authority should be able to interpret the results and decide whether or not a specific action is required.

Points to consider:

- Reporting requirements, if available;

- The completeness of the report, including transparency in presentation of methods, data and analytical tools used to draw conclusions;

- Accessibility to raw data accrued during the monitoring activities, taking into account information that may be confidential.

CHALLENGES IN THE IMPLEMENTATION OF A MONITORING STRATEGY

In the development (or evaluation) of a monitoring plan, it may become apparent that resource limitations or technical and scientific challenges may affect its effective implementation. Therefore, an analysis of the capacities and resources, human and financial, helps to ensure the maintenance and completion of the proposed monitoringplan. Amendments to the strategy may be required in some cases to ensure the monitoring strategy is efficient and cost-effective in relation to monitoring needs and expected outcomes.

Because changes or effects observed through monitoring may be a consequence of complex interactions of various biological and non-biological factors within the environment, it is essential that the monitoring activities are designed in a way to give meaningful information towards determining whether the observed effects and an LMO have a causal link (which may require further monitoring information or data).

Examples of challenges that may be encountered during the implementation of monitoring may include i) lack of capacity for the establishment of robust detection or identification methodologies, ii) determination of cause-effect relationships (causalities) between the LMO(s) and observed changes in the indicator(s) or parameter(s); and iii) the interpretation of monitoring results and relating them to further specific actions.

Annex 1

[Add graphic representation of the revised text]

Annex 2

Examples of monitoring in relation to protection goals/objectives

| Objectives | Indicator(s)/Parameter(s) | Example(s) of monitoring |
| --- | --- | --- |
| Reduction of levels of significant uncertainty of potential effects identified in the RA | Target organisms, Non-target organisms, environmental parameters, etc. | • Confirming host-range effects of target transgenic proteins, resistance development, • Confirming exposure routes or levels, if not maximized in the considerations of the risk assessment (worst case approach) |
| Impact on assessment endpoints or related indicators identified and evaluated in the RA | Target organisms, non-target organisms, environmental parameters, etc. | • Presence and population levels of key selected NTOs • Food web and predator/prey interactions of key selected NTOs at different trophic levels |
| Confirmation of in vivo exposure levels | Non-target organisms, etc. | • Direct or indirect uptake/exposure of NTOs to transgenic pesticidal proteins • Existence of weed species in herbicide tolerant (HT) fields • Accumulation of transgenic products in the soil |
| Impact on production systems in relation to sustainability | Functional organisms, key environmental services, etc. | • Pollination impacts • Pest control efficacy |
| Monitoring for scale-dependent effects | Wild and weedy relatives, HGT candidates | • Persistence of DNA or transgenic products in the soil • Frequency of gene transfer potential |
| Efficacy of risk management strategies | Weed populations, resistance development | • Efficacy of refugia strategies to delay resistance development of pesticide-producing crops by testing susceptibility of target pests • Recording weed populations in HT crop fields or adjacent areas |
| Conservation of biodiversity (including genetic diversity) and ecosystems | Primary producers (e.g. plants) and vertebrates (mammals, birds, fish, etc.), invertebrates (arthropods, fungi) with a focus on beneficial/functional organisms, important sources of genetic diversity or protected species | • Abundance and population changes • Resistance development, changes in pest prevalence or pathology • Effects of agrochemical usage associated with the LMO in indicator species • Developmental and fitness changes (direct and indirect) in indicator species • Host range or key behavioral changes in indicator species • Changes in dispersal, establishment and persistence in the LMO compared to the non-modified recipient organism • Landscape alterations • Outcrossing/hybridization with wild or weedy relatives |
| Soil quality and functional processes | Soil microbes and invertebrates (e.g. bacteria, fungi, and arthropods) particularly those providing key soil ecological services (nutrient cycling and decomposition) | • Population changes • Gene transfer frequencies • Organic compound changes • Effects of agrochemical usage associated with the LMO • Soil fertility changes • Changes to degradation processes • Soil erosion and compaction changes |
| Water quality and water pollution prevention | Physical and chemical pollutants in water, etc. | • Nutrient levels • Pollutants: pesticides, herbicides, etc. • Emission of transgenic product to water • Anoxia |
| Plant health | Plant diseases, pests and weeds, etc. | • Incidence of disease, pests and weeds • Pesticide usage |
| Human health (e.g. LMO handlers) | Handlers of LMOs or their products (e.g. farmers, research technicians, mill workers, etc.) | • Exposure analysis • Screens for toxic or immunogenic effects • Epidemiological surveys |
| Agroecosystem services | Floral and faunal indicators of functionality (pollinator populations, beneficial plant communities) | • Abundance • Foraging behaviors and pollination levels • Soil indicators |

Sources:

Food and Agriculture Organization of the United Nations. (2011). Biosafety resource book. Rome: FAO, Module B: Ecological Aspects and Module D: Test and Post-Release Monitoring of GMOs.

VDI-Guideline 4330 Part 1: Monitoring the ecological effects of genetically modified organisms, Genetically modified plants, Basic principles and strategies, 2006.

EFSA Panel on GMO; Scientific Opinion on guidance on the Post-Market Environmental Monitoring (PMEM) of genetically modified plants. EFSA Journal 2011;9(8):2316. [40 pp.]

See Article 1 of the Protocol.

COP-MOP decision BS-IV/11 (http://bch.cbd.int/protocol/decisions/decision.shtml?decisionID=11690).

See Roadmap “Overarching issues”, “Quality and relevance of information”.

See Roadmap “Overarching issues”, “ Identification and consideration of uncertainty”.

See Annex III pagraph 9 (a thru h)

See also considerations on “Quality and relevance of information” in the Roadmap.

See article 16(4) of the Protocol.

See article 21 of the Protocol.

This table includes a non-exhaustive list of examples that may be taken into account on a case-by-case basis, as appropriate, when developing a monitoring strategy.